ISONA Toolbox reference manual

ver 3.1

3.3.2006

A

AVE_IMAGES

Calculates and displays average images for each exposure level, both with and without trend

Function call:

ave_out = ave_images(dname,ext,Ngroup,index,trm,vis);

In:   dname       Directory name for data files

ext         	file name extension (e.g. 'x3f')

Ngroup      	Number of files in one exposure group (default = 10)

index       	Index numbers for exposure levels to be averaged

(default all)

trm         	trend removal; 1 = yes, 0 = no (default 0)

vis         	result plotting; 1 = yes, 0 = no (default 0, use

plot_ave_images for results)

Out:  ave_out   	cell array containg:

ch_ave1, ch_ave2 etc: 3D matrix of color channel averages with trend

Tch_ave1, Tch_ave2 etc: trendless averages for color channels

Function uses files.m to obtain names of the files in directory dname in time order. Number of different exposure levels is calculated from total number of files and number of frames per exposure level (Ngroup).

For each exposure level, Ngroup of image files are read and divided into color components. If trendless images are also required, the 2D trend removal filter used is defined by

[ba, aa] = besself(2, 0.01*2*pi);

[b, a] = impinvar(ba, aa, 1);

X = zeros(1001); X(501,501)=1;

H = filtsk(X, b, a); sH = sum(H(:));

H(:) = H/sH; b = b/sH;

and filtering is done with filtsk.m.

Each image is divided into color components and each component is averaged separately.

If the visualization parameter vis is set as one, function uses plot_ave_images.m to plot the resulting average images.

B

BASIC_ANALYSIS

Calculates mean, std, column averages and row averages from single image.

Function call:

[mI,sI,col_ave,row_ave] = basic_analysis(I);

In:   I		3D-image (if not given, uses uigetfile for file name)

Out:  mI           	Color component mean values

sI            	Color component std values

col_ave   	Color component column averages

row_ave 	Color component row averages

This function is for quick analysis of a single image. Mean and std values give an idea of signal and noise levels, and row and column averages reveal possible trends in signal level as a function of sensor row/column.

D

DC_SHOTNOISE

Analyses dark current shot noise for different exposure levels

Function call: out = dc_shotnoise(dname,ext,Ngroup,index,ave_in);

In:   dname       Directory name for data files

ext         	file name extension (e.g. 'x3f')

Ngroup      	Number of files in one exposure group (default = 100)

index       	Index numbers for exposure levels to be averaged

(default all)

ave_in      	cell array (output from ave_images) containing

ave_ch1, ave_ch2 etc; average dark images for color components for each exposure level in matrix

Out:  out	cell array containing shot_ch1, shot_ch2 etc;

3D matrices of pixel-by-pixel dc shot noise variances

Given the average color component dark images for each exposure level (exposure time) to be analyzed, dc_shotnoise calculates pixel-by-pixel temporal dark noise variance.

DCSHOT_DIST

Calculates dark current shot noise for single exposure level for given (or random) image points

Function call:

out = dc_shotnoise(dname,ext,explev,Ngroup,index,ave_in);

In:   dname	Directory name for data files

ext         	file name extension (e.g. 'x3f')

explev      	index of exposure level in directory dname to be analysed

Ngroup      	Number of files in one exposure group (default = 100)

index       	Index numbers for image points to be analysed

(default = ten random points)

ave_in      	cell array (output from ave_images) containing

ave_ch1, ave_ch2 etc; average dark images for color components for wanted exposure level in matrix

Out:  out         	cell array containing shot_ch1, shot_ch2 etc;

[Ngroup x length(index)] matrices of pixel-by-pixel dc

shot noise values

This function is designed for analyzing dark current shot noise distributions from large number of frames pixel-wise. In order to save memory, only given set of pixels are analysed.

F

FILES

Searces names of files of type typ in given directory (or list of directories) D

Function call:  Nam = files(D, typ, opt);

In:  D	      root directory (default .)

typ	      type (or list of types) of files to search (default all)

(type is considered case insensitive)

opt	option for filenames listed by name ('n', default) or     date/time ('t')

Out: Nam   list of names of found files

FILTSK

Filters image using forward and backward separate kernel 1D-filters

Function call:  Ir = filtsk(I, b, a);

In:  I      matrix (image)

b,a  1D-filter coefficients

Out: Ir    filtered image

Note: filtsk uses filtfilt from Signal Processing Toolbox

FPN

Analyses vertical and horizontal fixed pattern noise (SMIA 1.0 characterization 5.2, 5.3)

Function call:    [VFPN,HFPN] = fpn(FSD,ave_in);

In:   	FSD    	Full scale deflection for camera

(max pixel output - min pixel output)

ave_in  	cell array (output from ave_images) containing

ave_ch1, ave_ch2 etc; average color components

Out:  	VFPN   	Vertical fixed pattern noise for each color (% of FSD)

HFPN   	Horizontal FPN for each color (% of FSD)

Vertical fixed pattern noise (VFPN) is a measure of the vertical strips and banding that can be seen in an image especially at low light levels. VFPN level is mathematically described as the RMS of the column-to-column difference of the image.

Correspondingly, horizontal fixed pattern noise (HFPN) is a measure of the horizontal strips and banding that can be seen in an image especially at low light levels. HFPN level is mathematically described as the RMS of the row-to-row difference of the image.

VFPN and HFPN are preferably analyzed from average of several images, but this analysis can also be done for a single image.

G

GLOBMAX

Finds a global maximum value of N-D array X.

Function call:   [y, loc] = globmax(X);

In:      X     N-D array X

Out:   y     global maximum of the array

loc   location [row, col, dim] of maximum

GLOBMIN

Finds a global minimum value of N-D array X.

Function call:   [y, loc] = globmin(X);

In:      X     N-D array X

Out:   y     global minimum of the array

loc   location [row, col, dim] of maximum

H

HISTMULTI5

Calculates multidimensional histograms

Function call:   [n,x,nbins]=histmulti5(y,bins)

In:  y	multidimensional data; every row in y represents a different data point and the columns are the different coordinates.

For one-dimensional data you should use a column-vectors.

bins: a rowvector => This is the number of bins in that coordinate.

bins: a matrix => The lower limits for the different bins as produced in x.

Pad at end of the columns with NaN if different number of bins

in different coordinates (and unequal bins).

The upper limits are the lower limit for the next bound and inf for

the last one.

The default is that bins=[10 ... 10];

Out:

n - a multidimensional vector with counts.

x - Corresponds to the bins-matrix (with NaN-padding if necessary).

The algorithms are constructed to be very fast, and can be superior

to hist even for one-dimensional histograms.

It uses a mex-file to get good performance, and the first time it

automatically compiles the mex-file. If you have trouble compiling

c-files please compile the c-file by hand or erase it.

Rewritten for Matlab 5 by Hans Olsson, Hans.Olsson@dna.lth.se

Original Matlab 4 routine written by Hans Olsson, Hans.Olsson@dna.lth.se.

Inspired by hist2 written by Kirill K. Pankratov, kirill@plume.mit.edu

Which in turn was based on routines written by Hans.Olsson@dna.lth.se

I

IMMULTI

Displays multiple image frames as rectangular montage.

Function call:   h = immulti(I,lims)

In:   I       M-by-N-by-K image matrix

lims  limits for gray level scaling (default: [globmin(I) globmax(I)])

Out:  h    handle to the image object.

IMMULTI_HIST

Displays multiple histogram image frames as rectangular montage.

Function call:   h = immulti_hist(I,blocks,lims)

In:   I             M-by-N-by-K histogram image matrix

blocks    number of blocks (N_vertical x M_horizontal, default                    sqrt(size(I,3)))

lims        limits for gray level scaling (default: [globmin(I) globmax(I)])

Out: h             handle to the image object.

ISVALIDEXT

Checks out if the file extension is valid for function imread.

Function call: [out,FMT] = isvalidext(ext);

In:  	ext     	file extension

Out: 	out     	1 for valid extension, 0 otherwise

FMT     	file format string for imread

N

NOISE_ANALYSIS

Calculates noise components according to ISO standard ISO 15739

Function call:

[s_ave,var_diff,s_temp,s_fp,mC,SNR_temp,PRNU,PRNU_lev,out]

= noise_analysis(dname,ext,exposure,offset,smax,Ngroup);

In:   dname 	Directory name for data files

ext         	file name extension (e.g. 'x3f')

exposure    Input exposure values [lux*s] or scene log luminance

values [cd*s/m^2]

offset      	camera offset level

smax	maximum signal level for camera

Ngroup      	Number of files in one exposure group (default = 10)

Out:  s_ave      	standard deviations of average color components (columns)

var_diff   	means of variances of differenced color components (columns)

s_temp     	standard deviations of temporal noise for color components

s_fp       	standard deviations of the fixed pattern noise

mC         	mean values of average color components at each exposure level

SNR_temp 	signal-to-temporal noise ratio at each exposure level

PRNU       	photo-response non-uniformity

PRNU_lev  	photoresponse non-uniformity determination level

out        	cell array containing:

ave_ch1, ave_ch2 etc; 3D-matrices of average color component values (one for each exposure level,

64 x 64 matrix from the middle of the image)

std_ch1, std_ch2 etc: pixel-by-pixel STDs of color components at each exposure level

This function carries out the noise component analysis according to standard ISO 15739 for each measured exposure level. The method used corresponds to “Method using eight images” presented in the standard, only the number of images used is arbitrary.

Firstly, the average color components (R_ave etc.) are calculated, then the pixel-by-pixel STDs (std_R etc.) for each color component. Mean values of average color components (mC) are calculated. Columns in mC correspond to color components (red, green1, green2, blue) and rows correspond to different exposure levels.

Standard deviations of average color components (s_ave) are calculated for each exposure level (ave in standard). Then, in order to remove fixed pattern noise, the difference of each image and the average image is calculated. The next step is to calculate the STDs of the code values of the difference images to give  , where j = 1…Ngroup.

In the next step the mean of all squared STDs of the difference images (var_diff) is calculated to give

The STD of the temporal noise (s_temp) is then calculated as

The STD of the fixed pattern noise(s_fp)  is calculated as

O

OECF

Calculates opto-electronic conversion function (OECF) according to ISO 14524

Function call:

[val,out] = oecf(dname,ext,exposure,Ngroup,pixarea,vis);

In:   dname	Directory name for data files

ext         	file name extension (e.g. 'x3f')

exposure    	Input exposure values [lux*s] or scene log luminance

values [cd*s/m^2]

Ngroup      	Number of files in one exposure group (default = 10)

pixarea     	pixel area used for calculation,

[min_row max_row min_col max_col]

(default 64 x 64 pixels in the middle of the image)

vis         	result plotting; 1 = yes, 0 = no (default 0, use

plot_oecf for results)

Out:  val         	logarithmic exposure values

out         	cell array containg:

ch_mean1, ch_mean2 etc; means of color channels of average images

Camera OECF represents the camera output signal as a function of input exposure values [lux*s] or scene log luminance values [cd*s/m^2]. Function OECF reads in the images of different exposure levels and calculates color component averages.

If the visualization parameter vis is set as one, function uses plot_oecf.m to plot the resulting OECF curve.

P

PIX_DEFECT

Analyses image to find possible defective pixels (SMIA 1.0 characterization 2.2.3 and 2.2.4)

Function call:    [I_error,loc] = pix_defect(I,k,FSD,theta);

In:   I           3D-image (R,G1/Gr,B,G2/Gb) (if not given, uses uigetfile  for file name)

k          Kernel size (+/-k pixels around analyzed pixel, default 1)

FSD     Full scale deflection for camera

theta    threshold for defectiveness as % of FSD (default 15)

Out:  I_error   Error image

loc         Location and value of defective pixels in each color component (struct)

This routine detects pixel defects on the image by recording the difference between the local pixel value and the kernel Kk mean. For each pixel A(x,y), Kk = A(x-k:x+k,y-k:y+k), and

Then, if , pixel is interpreted as defective. Output parameter loc is structure containing defective pixel locations and values for each color component.

PLOT_AVE_IMAGES

Plots average images

Function call:    plot_ave_images(trm,ave_in);

In:   trm        	trendless averages included, 1 = yes, 0 = no

ave_in      	cell array containg:

ch_ave1, ch_ave2 etc: 3D matrix of color channel averages with trend

Tch_ave1, Tch_ave2 etc: trendless averages for color channels (output from ave_images)

Plot_ave_images uses function immulti.m to plot average color components from different exposure levels as a rectangular montage.

PLOT_NOISE_RESULT

Plots noise component analysis results

Function call:

plot_noise_result(exposure,s_ave,s_temp,s_fp,…

SNR_temp,PRNU,PRNU_lev,arg_in);

In:   exposure	Input exposure values [lux*s] os scene log luminance values [cd*s/m^2]

s_ave      	standard deviations of average color components (columns)

s_temp     	standard deviations of temporal noise for color components

s_fp       	standard deviations of the fixed pattern noise

SNR_temp 	signal-to-temporal noise ratio at each exposure level

PRNU       	photo-response non-uniformity

PRNU_lev  	photoresponse non-uniformity determination level

arg_in     	cell array containing:

ave_ch1, ave_ch2 etc; 3D-matrices of average color component values (one for each exposure level,

64 x 64 matrix from the middle of the image)

std_ch1, std_ch2 etc: pixel-by-pixel STDs of color components at each exposure level

(output from noise_analysis)

Creates noise analysis result plots from the results of function noise_analysis.m.

PLOT_OECF

Plots opto-electronic conversion function (OECF)

Function call:    plot_oecf(ext, val, mean_in);

In:   ext         	file name extension (e.g. 'x3f')

val         	logarithmic exposure values

mean_in	cell array containing

ch_mean1, ch_mean2 etc: Means of color components of average images

Creates OECF curve plot from the results of function OECF.m.

PLOT_PTC

Plots photon transfer curve (PTC)

Function call:    plot_ptc(ext, S, std_S, std_FPN);

In:   ext           file name extension (e.g. 'x3f')

S             Averages of mean red, green and blue digital values     for each exposure level

std_S      Averages of STDs of differenced red, green and blue  components

std_FPN  Fixed pattern noise STDs for color components

Creates PTC curve plot from the results of function ptc.m. Curve is plotted separately for each color component, and for each component the curves of fixed pattern noise, temporal noise and total noise are plotted.

PTC

Calculates photon transfer curve (PTC) according to Janesick: "Scientific Charge-Coupled Devices"

Function call:

[S, std_S, std_FPN, K, RN, FWC, DR, offset] =

ptc(dname, ext, Ngroup, pixarea, vis);

In:   dname	Directory name for data files

ext         	file name extension (e.g. 'x3f')

exposure    Input exposure values [lux*s] or scene log luminance values [cd*s/m^2]

Ngroup      	Number of files in one exposure group (default = 10)

pixarea     	pixel area used for calculation,

[min_row max_row min_col max_col]

(default 64 x 64 pixels in the middle of the image)

vis         	result plotting; 1 = yes, 0 = no (default 0, use

plot_ptc for results)

Out:  S           	Averages of mean red, green and blue digital values for each exposure level

std_S       	Averages of STDs of differenced red, green and blue components

std_FPN     Fixed pattern noise components

K           	camera gain constant [e-/DN] (mean and std

for each color component plus number of estimation                    points, nc by 3 matrix)

RN          	Read noise [rms e-]  (nc by 1 matrix)

FWC         	Full well capacity [e-/pixel] (nc by 1 matrix)

DR          	Dynamic range [dB] (nc by 1 matrix)

offset      	signal electrical offset value

NOTE: nc = number of color components in image

Photon transfer process for a CCD is represented in Figure 1. Photon transfer curve (Figure 2) is a loglog plot of signal noise as function of signal level. Photon transfer method is used to evaluate numerous image sensor parameters in absolute terms. These parameters include read noise, dark current, quantum yield, full well, linearity, pixel nonuniformity, sensitivity, signal-to-noise, offset, and dynamic range. Photon transfer also gives the conversion constant K used to convert relative digital numbers (DN) generated by the camera into absolute physical units of electrons. This conversion constant is referred to as “camera gain constant,” expressed in e/DN.

Figure 1

Fixed pattern noise is the dominant noise component, and removing the pixel nonuniformity makes it easier to determine the shot noise part of the curve. Pixel nonuniformity is eliminated by differencing, pixel by pixel, two identical images taken back to back at the same exposure level. To improve statistics, three or more (preferably much more) exposures at the same signal level can be taken. The noise is determined by differencing each image with the average of Ngroup images and calculating the mean standard deviation of these differenced images using 64 x 64 pixel areas from the centre of the image.

Figure 2

If the signal variance (noise level) is considerably higher than the read noise level (in this function at least 2 times the read noise variance), camera gain constant K can be calculated from equation

where S(DN) is signal as digital numbers,  is signal variance and  is read noise floor variance.

R

READRAW

Reads image from raw digital camera file

Function call:  I = readraw(fname, opt);

In:    fname	any 'raw' filename

Out: I	   (uint16 or uint8) image

opt     read options character string (default none - uint8 true-              color)

'L' - 16-bit linear true-color

'O' - raw binary, 'c' - color corrected raw binary

'rM' - include black margins into raw binary

Note: Uses dcraw_any.exe (modified from Dave Coffins dcraw by HI) or imread.m

For more information on reading digital camera raw data, see

http://www.cybercom.net/~dcoffin/    (referred 12.5.2005)

REGH

Computes regression from 2D-histogram h.

Function call:    [mb_a, vb_a] = regh(h, a, b);

In:	h	2D-histogram counts h

a, b    	variable axes a and b

Out:	mb_a	conditional mean of b on a

vb_a	conditional variance of b on a

n	degrees of freedom

S

SINGLE_IMAGES

Displays individual images, both with and without trend

Function call:    out = single_images(dname,ext,index,trm);

In:   dname       Directory name for data files

ext         	file name extension (e.g. 'x3f')

index       	Index numbers for files to be shown

(time order in directory dname) or file names in cell

array

trm         	trend removal yes = 1, no = 0 (default = 0)

Out:  out         	cell array containg:

ch1, ch2 etc: 3D matrix of color components with trend

Tch1, Tch2 etc: trendless averages for color

Reads in images, removes trend if trendless images are required, and shows the images in separate windows.

START_ISONA

Initializes ISONA GUI.

Script call:    start_ISONA

X

XCOR

Computes the correlation function between a signal couple.

Function call:       r = xcor([x y], md);

In:    [x y]   two-variate signal formed by signals to be correlated.

(If given univariate signal, autocorrelation is calculated.)

md    maximum delay calculated (default maximum possible)

Out:  r       correlation function of delay values -md...md

Function gr_imbalance uses xcor to calculate the correlation between green lines.